1. Initial review of the Lakeside K 502 sample

I checked the supplied Lakeside K 502 EMD file and the channel-mapping workbook. The CSV contains phase channels plus individual end-use channels such as ACs, geysers, refrigerator, lighting, oven and washing machine. Lakeside K 502

A few useful observations from the sample:

| Check | Lakeside K 502 result | Implication |
| --- | --- | --- |
| Records | 110,145 | Strong high-frequency dataset |
| Period | May 2025–May 2026 | Full seasonal cycle |
| Theoretical 5-min slot coverage | ~98.5% | Very good |
| Observed intervals that are exactly 5 min | ~99.84% | Excellent temporal consistency |
| Duplicate timestamps | 0 | Good |
| Missing values by channel | ~1.0–1.45% | Manageable |
| Phase vs mapped appliance correlation | >0.9997 | Channel mapping is highly consistent |
| Mean phase/appliance reconciliation error | <~0.0022 | Very strong electrical reconciliation |

There is, however, a critical data-cleaning issue. Some individual channels contain small negative readings, while isolated values reach implausibly high levels—for example, the raw R-phase reaches above 40 in the recorded units. I would not simply convert every negative value to its absolute value. For forecasting and energy analysis, the better approach is to identify sensor zero-offset/polarity errors, meter spikes and physically impossible readings, then correct or flag them based on phase reconciliation and appliance-specific limits.

Also, the CSV channel headings do not specify the measurement unit. Lakeside K 502 The data behave like instantaneous power measurements, but this should be verified from the Schneider EMD configuration before converting them to kWh.

2. Recommended analytics framework for Lodha Palava

I would develop the project around 14 analytics modules.

| # | Analytics | What it tells you | Recommended methodology | Research basis |
| --- | --- | --- | --- | --- |
| 1 | Apartment Energy Fingerprint | Typical 24-hour consumption pattern of every apartment | 96 × 15-min daily profile; weekday/weekend/season clustering; normalized and absolute profiles | Residential load-shape clustering is well established for identifying distinct household behaviour. IDEAS/RePEc |
| 2 | Energy Lifestyle / Behaviour Clustering | Apartments that behave similarly even if flat size differs | K-means/HDBSCAN on normalized load shapes, appliance shares, timing and variability | Kwac et al. developed household segmentation using smart-meter load-shape dictionaries; clustering approaches have also been demonstrated on residential demand profiles. ResearchGate |
| 3 | Weather-normalized Consumption | Whether high consumption is actually caused by hotter/humid weather | Change-point regression / GAM / LightGBM using outdoor T, RH and calendar variables | Weather and floor area have been shown to be major determinants of residential electricity use; change-point regression is also an established weather-normalization method. IDEAS/RePEc |
| 4 | Cooling Sensitivity Index | Which apartment's AC consumption rises fastest when outdoor temperature rises | AC kWh vs cooling-degree-hours, outdoor enthalpy and humidity; apartment-specific change point | Derived from weather-normalized building energy models and residential energy determinant research. IDEAS/RePEc |
| 5 | Indoor–Outdoor Thermal Buffering | How effectively each apartment resists outdoor heat | ΔT, thermal lag, daytime temperature rise rate, overnight decay rate | Grey-box thermal models can identify building thermal dynamics from measured data. ScienceDirect |
| 6 | Apartment Thermal Time Constant | Whether the apartment heats up quickly or retains cooling | RC/grey-box model from Tin, Tout and AC power | Bacher & Madsen showed how grey-box models can extract building heat dynamics from measurements. ScienceDirect |
| 7 | AC Cooling Effectiveness | How much indoor-temperature improvement is obtained per unit AC energy | Tin response before/during/after AC operation, normalized for Tout/RH | Smart-residential studies have successfully predicted indoor T/RH using weather and HVAC/thermostat information. MDPI |
| 8 | Humidity / Latent Cooling Performance | Whether AC is effectively controlling humid conditions | RH/dew-point/humidity-ratio response during AC operation | Indoor temperature and humidity models have been demonstrated using NARX and weather/thermostat information. MDPI |
| 9 | Occupancy / Activity Proxy | Probable occupied, active and inactive periods without separate PIR sensors | appliance transitions + lighting + AC + cooking + washing patterns | Electricity consumption can provide useful occupancy information; studies report successful residential occupancy inference from meter data. FIS Universität Bamberg |
| 10 | Baseload & Phantom Consumption | Energy consumed continuously irrespective of occupancy | Night minimum load, persistent device load, quantile baseline | Residential research has specifically examined determinants of minimum/idle consumption. IDEAS/RePEc |
| 11 | Peak Demand Driver Analysis | Which appliances create apartment peaks | coincidence analysis, SHAP contribution, simultaneous appliance states | Appliance-level monitoring allows peak drivers to be attributed directly rather than inferred |
| 12 | Flexible Load / Demand Response Score | Loads that can potentially be shifted without major comfort impact | Geyser, washing machine and selected AC events; event-duration and time-window analysis | Current residential energy research increasingly uses appliance usage patterns for demand response; recent work explicitly combines appliance identification, ambient temperature and residential DR. ScienceDirect |
| 13 | Matched-Apartment Performance Benchmark | Whether a flat uses more energy than similar flats after correcting for conditions | Match by area, BHK, orientation, floor and weather; calculate Actual/Expected ratio | Research shows climate, floor area, appliance stock and behaviour must be accounted for when comparing residential electricity consumption. IDEAS/RePEc |
| 14 | Predictive & Prescriptive Analytics | What will happen tomorrow and what operational action may reduce consumption | Global forecasting + apartment personalization + probabilistic prediction | Residential LSTM, probabilistic forecasting, transfer learning and GNN approaches provide a strong research basis. HKU Scholars Hub |

3. Some analytics that would be particularly innovative for this project

A. Apartment Cooling Performance Signature

Instead of simply saying:

Apartment X used 120 kWh of AC energy.

calculate something like:

\[ CPI = \frac{\text{AC Energy}} {\text{Cooling Degree Hours}} \]

and improve it further using apartment area:

\[ CPI_A = \frac{E_{AC}} {A \times CDH} \]

where:

- \(E_{AC}\) = AC energy

- \(A\) = apartment floor area

- CDH = Cooling Degree Hours

This permits a meaningful comparison between, for example, a south-facing 78.6 m² flat and a smaller north-facing apartment.

Even better, estimate each apartment's balance-point temperature from the measured data rather than assuming 24°C or 26°C.

4. Thermal Buffering Index

Because you have indoor and outdoor temperatures, calculate:

\[ \Delta T(t)=T_{out}(t)-T_{in}(t) \]

during periods where AC is OFF.

Then determine:

- maximum indoor/outdoor differential;

- temperature rise per hour;

- lag between peak outdoor and peak indoor temperature;

- overnight cooling rate;

- daytime thermal stability.

A thermally better-performing apartment should generally exhibit a slower indoor response to outdoor temperature excursions.

A grey-box model can be expressed approximately as:

\[ T_{in,t+1} = aT_{in,t} + (1-a)T_{out,t} - bP_{AC,t} + cX_t+\epsilon_t \]

where \(X_t\) can contain humidity, time-of-day and other effects.

The parameter \(a\) becomes related to the apartment's thermal inertia, while \(b\) represents the observed influence of AC power.

This approach is directly aligned with grey-box building heat-dynamics research. ScienceDirect

5. Cooling Effectiveness rather than just AC energy

Because AC channels are independently monitored, you can quantify:

AC effectiveness

\[ CE = \frac{-\Delta T_{in}} {E_{AC}} \]

but this should then be normalized against outdoor conditions.

For example, compare two AC events:

| Apartment | AC energy | Outdoor | Indoor start | Indoor after 1 h |
| --- | --- | --- | --- | --- |
| A | 1.1 kWh | 34°C | 30°C | 26°C |
| B | 1.1 kWh | 34°C | 30°C | 28.5°C |

Both consumed the same electricity, but their thermal result is very different.

That difference may indicate:

- different AC capacities/efficiencies;

- solar exposure;

- envelope differences;

- infiltration;

- occupant behaviour;

- door/window opening;

- sensor location;

- AC maintenance condition.

This would be one of the strongest analytical outputs available from your dataset.

6. Comfort–Energy Efficiency Index

You can create a metric such as:

\[ CEI = \frac{\text{Comfortable occupied hours}} {\text{AC kWh}} \]

The objective is not simply “lowest AC energy”.

A flat consuming very little AC energy while spending most hours at 31°C is not necessarily performing better.

This allows a dashboard quadrant:

|  | Low Energy | High Energy |
| --- | --- | --- |
| Good indoor condition | Best-performing | Cooling inefficient |
| Poor indoor condition | Under-cooled / behaviour issue | Worst-performing |

Because you only have temperature and RH, I would initially call this Indoor Environmental Condition Efficiency, rather than claim full PMV/ASHRAE thermal comfort. Full thermal comfort assessment also requires variables such as air velocity, clothing and metabolic rate. Residential comfort research similarly emphasizes that dry-bulb temperature alone does not fully characterize thermal comfort. MDPI

7. Occupancy should be treated as an “activity probability”

You have enough appliance information to derive useful occupancy-related behaviour.

For example:

- lighting transition;

- geyser ON;

- kitchen loads;

- washing machine operation;

- multiple appliances changing simultaneously;

- AC operation;

- low night baseload.

Research has demonstrated occupancy inference from electricity data, including large-scale residential datasets. FIS Universität Bamberg

However, because you do not have ground-truth occupancy labels, I recommend calling it:

Household Activity Probability

rather than absolute occupancy.

Example:

\[ P(Activity_t=1)= f( Lights_t, Kitchen_t, Geyser_t, AC_t, WM_t, \Delta P_t ) \]

This can then become an input into the load-forecasting model.

8. Matched apartment benchmarking

This is particularly valuable because your 17 apartments differ in:

- area;

- floor;

- building type;

- apartment type;

- orientation;

- duration.

A simple kWh/apartment comparison would therefore be misleading.

Instead predict:

\[ \widehat E_{i,d} = f( Area_i, Floor_i, Orientation_i, FlatType_i, BuildingType_i, Weather_d, Calendar_d ) \]

Then define:

\[ Performance\ Index = \frac{Actual\ Energy} {Expected\ Energy} \]

Example:

- 0.82 = uses 18% less than expected;

- 1.00 = approximately expected;

- 1.27 = uses 27% more than expected.

This gives you a much more defensible apartment performance score.

Kavousian et al.'s analysis of 1,628 households found that climate, location, floor area, appliance characteristics and occupant behaviour can materially affect residential electricity consumption. IDEAS/RePEc

9. Prediction methodology I recommend

The most important recommendation is:

Do not develop 17 completely independent prediction models.

Some apartments have only 9 months of data. Even the 13-month apartments contain only one full annual seasonal cycle.

Instead use a Global Residential Forecasting Model with Apartment Personalization.

Research increasingly supports learning common patterns across households and transferring/shared learning between residences. IJCAI

The prediction becomes:

\[ \hat P_{i,t+h} = f( \underbrace{P_{i,t-L:t}}_{\text{historical load}}, \underbrace{A_{i,t-L:t}}_{\text{appliances}}, \underbrace{T_{in},RH_{in}}_{\text{indoor}}, \underbrace{T_{out},RH_{out}}_{\text{weather}}, \underbrace{C_t}_{\text{calendar}}, \underbrace{M_i}_{\text{apartment metadata}} ) \]

where:

- \(i\) = apartment;

- \(t\) = present time;

- \(h\) = prediction horizon;

- \(L\) = historical look-back window;

- \(M_i\) = area, floor, orientation, apartment type etc.

10. Data synchronization

Because EMD = 5 minutes and Bridgethings/weather = 15 minutes, I recommend maintaining two analytical layers.

Electrical layer — 5 minutes

Use for:

- appliance ON/OFF detection;

- peak demand;

- simultaneous appliance use;

- AC cycling;

- geyser events;

- refrigerator duty cycle;

- washing-machine cycle analysis;

- anomalies.

Integrated energy + thermal layer — 15 minutes

Aggregate three EMD samples.

If Schneider values are confirmed as kW:

\[ E_{15} = \sum_{j=1}^{3}P_{5,j} \times \frac{5}{60} \]

and

\[ \bar P_{15} = \frac{P_1+P_2+P_3}{3} \]

Then join:

Timestamp | Apartment | Total kW | AC kW | appliance kW | Tin | RHin | Tout | RHout | metadata

11. Forecasting features

For every 15-minute timestamp I would generate:

| Feature family | Variables |
| --- | --- |
| Historical load | t−15m, t−30m, t−1h, t−3h, t−24h, t−48h, t−7d |
| Rolling statistics | 1h/3h/6h/24h mean, max, SD |
| Time | hour, weekday, weekend, month, holiday |
| Outdoor | T, RH, dew point, humidity ratio, enthalpy |
| Indoor | Tin, RHin |
| Thermal | Tout−Tin, RHout−RHin, Tin rate-of-change |
| AC | AC power, ON/OFF, duty cycle, previous run duration |
| Appliances | geyser, refrigerator, washing machine, oven, lighting |
| Building | area, BHK, floor, orientation, building type |
| Behaviour | activity probability, evening-use index, night baseload |

Use cyclical encodings for hour/day:

\[ Hour_{sin}=\sin(2\pi h/24) \]\[ Hour_{cos}=\cos(2\pi h/24) \]

12. Different targets need different prediction models

I would not use one algorithm for everything.

| Prediction | Horizon | Preferred starting model |
| --- | --- | --- |
| Total apartment load | 15 min–1 h | LightGBM / XGBoost |
| Total load profile | next 24 h | Global LightGBM → LSTM/TFT comparison |
| Daily consumption | next day | Gradient boosting |
| AC ON/OFF | 15 min–6 h | Classification model |
| AC load | 15 min–6 h | Two-stage / hurdle model |
| Indoor temperature | 15 min–6 h | Grey-box RC + NARX/LSTM comparison |
| Indoor RH | 15 min–6 h | NARX / gradient boosting |
| Peak demand | next day | Quantile regression |
| Monthly consumption | 1 month | Daily predictions aggregated using forecast weather |

For residential short-term load forecasting, LSTM has an established research basis, including individual-customer forecasting. HKU Scholars Hub Deep learning has also been applied to customer/building energy prediction more broadly. ScienceDirect

More recent work is moving toward Transformer/weather-integrated architectures and spatio-temporal models across multiple households. Springer

13. AC prediction requires a special model

Residential AC load is strongly zero-inflated.

Therefore, instead of directly predicting AC power:

Model 1 — Will the AC switch ON?

\[ p_t=P(AC_{ON}=1|X_t) \]

Using:

- Tin;

- Tout;

- RH;

- time;

- previous AC state;

- historical behaviour;

- apartment characteristics.

Model 2 — If ON, how much power?

\[ \hat P_{AC|ON}=g(X_t) \]

Therefore:

\[ E[P_{AC}] = P(ON)\times E[P_{AC}|ON] \]

This is generally much more sensible than forcing one regression model to learn thousands of zero readings.

The same event-based approach can be used for:

- geysers;

- oven;

- washing machine.

14. Probabilistic prediction is more useful than one number

A residential apartment is highly stochastic.

Instead of:

Tomorrow's energy = 10.7 kWh

provide:

P10 = 8.1 kWh
P50 = 10.6 kWh
P90 = 14.2 kWh

This is particularly useful for:

- peak-demand warnings;

- demand response;

- community transformer planning;

- energy budgeting.

Conformalized Quantile Regression provides a strong methodological basis for calibrated prediction intervals. arXiv

More importantly for your case, a 2023 Applied Energy study combined probabilistic forecasting, causal machine learning and thermal-load separation for residential communities, which is very closely aligned with what you can do using AC + weather data. ScienceDirect

15. Cross-apartment prediction is where the dataset becomes particularly interesting

You have 17 apartments all exposed to broadly the same Palava climate but with different:

- orientations;

- floors;

- areas;

- layouts;

- behavioural profiles.

We can create an Apartment Similarity Network.

For example:

\[ Similarity(i,j) = f( Area, Orientation, Floor, FlatType, LoadShape, CoolingSensitivity ) \]

Then a low-data apartment can borrow information from similar apartments.

This is conceptually aligned with graph-based and transfer-learning residential forecasting research. Lin and Wu specifically proposed transferring information between residential houses using graph neural networks. IJCAI More recent residential forecasting research uses dynamic spatio-temporal graphs to capture stable and changing relationships between households. ScienceDirect

For only 17 apartments, however, I would make this Phase 2 R&D, not the first production model.

16. Validation methodology

This is important.

Do not randomly split the 5-minute records into training/testing. That causes temporal leakage and can produce unrealistically good results.

Use rolling time-based validation.

For a 13-month apartment, for example:

Train → Validate → Test

May 2025–Feb 2026 → Mar 2026 → Apr–May 2026

Then repeat using rolling-origin validation.

For the complete project I would additionally perform:

Leave-One-Apartment-Out Validation

Train the global model on 16 apartments and predict the 17th.

This tells you whether the method can generalize to a new flat.

That becomes extremely useful if Lodha later wants to scale from 17 monitored apartments to hundreds or thousands of apartments.

17. Recommended performance metrics

Do not rely on MAPE alone because many appliance readings are zero.

For energy/power:

\[ MAE=\frac{1}{n}\sum |y-\hat y| \]\[ RMSE= \sqrt{\frac{1}{n}\sum(y-\hat y)^2} \]

Use:

MAE + RMSE + WAPE + CV(RMSE).

For probabilistic predictions:

- Pinball Loss;

- P10/P90 coverage;

- interval width.

For indoor temperature:

- MAE °C;

- RMSE °C.

For AC ON/OFF:

- Precision;

- Recall;

- F1;

- ROC-AUC.

18. Recommended dashboard structure

Ultimately, I would arrange the Lodha Palava analytics platform into six analytical pages:

- Apartment Energy Profile — total energy, end-use contribution, hourly/day/season profiles.

- Appliance Intelligence — AC, refrigerator, geyser, lighting, washing-machine patterns, duty cycles and anomalies.

- Energy–Weather–Comfort — indoor/outdoor conditions, cooling sensitivity, thermal lag, AC effectiveness.

- Apartment Performance Benchmark — Actual vs Expected, kWh/m², weather-normalized score, matched-apartment comparison.

- Prediction & Alerts — next-hour, next-day, peak demand, AC probability and indoor-temperature forecasts.

- Recommendations & Opportunities — abnormal baseload, excessive AC runtime, unnecessary simultaneous use, flexible load and behavioural opportunities.

19. Core research references for your methodology

These are the papers I would actually cite in a technical methodology/report.

| Research | Relevance to your Lodha analytics |
| --- | --- |
| Chicco, G. (2012), “Overview and performance assessment of the clustering methods for electrical load pattern grouping,” Energy. DOI: 10.1016/j.energy.2011.12.031 | Load-shape clustering and household segmentation. IDEAS/RePEc |
| Rhodes et al. (2014), “Clustering analysis of residential electricity demand profiles,” Applied Energy. DOI: 10.1016/j.apenergy.2014.08.111 | Seasonal residential load-profile clustering. ScienceDirect |
| Kwac, Flora & Rajagopal (2014), “Household Energy Consumption Segmentation Using Hourly Data,” IEEE Transactions on Smart Grid. DOI: 10.1109/TSG.2013.2278477 | Behavioural/load-shape segmentation of households. ResearchGate |
| Kavousian, Rajagopal & Fischer (2013), “Determinants of residential electricity consumption,” Energy. DOI: 10.1016/j.energy.2013.03.086 | Weather, area, appliance stock and behavioural effects. IDEAS/RePEc |
| Bacher & Madsen (2011), “Identifying suitable models for the heat dynamics of buildings,” Energy and Buildings. DOI: 10.1016/j.enbuild.2011.02.005 | Apartment thermal-response / grey-box modelling. ScienceDirect |
| Lou et al. (2020), “Smart Wifi Thermostat-Enabled Thermal Comfort Control in Residences,” Sustainability. DOI: 10.3390/su12051919 | Indoor temperature/RH prediction from residential thermal/weather data. MDPI |
| Kleiminger et al. (2013), “Occupancy Detection from Electricity Consumption Data.” DOI: 10.1145/2528282.2528295 | Electricity-based household activity/occupancy inference. FIS Universität Bamberg |
| Razavi et al. (2019), “Occupancy detection of residential buildings using smart meter data,” Energy and Buildings. DOI: 10.1016/j.enbuild.2018.11.025 | Large-scale validation of smart-meter occupancy inference. ScienceDirect |
| Kong et al. (2019), “Short-Term Residential Load Forecasting Based on LSTM Recurrent Neural Network,” IEEE Transactions on Smart Grid. DOI: 10.1109/TSG.2017.2753802 | Individual residential load forecasting. HKU Scholars Hub |
| Mocanu et al. (2016), “Deep learning for estimating building energy consumption.” DOI: 10.1016/j.segan.2016.02.005 | Deep-learning energy prediction methodology. ScienceDirect |
| Romano, Patterson & Candès (2019), “Conformalized Quantile Regression.” | Reliable P10/P50/P90 prediction intervals. arXiv |
| Applied Energy (2023), “Total and thermal load forecasting in residential communities through probabilistic methods and causal machine learning.” DOI: 10.1016/j.apenergy.2023.121783 | Especially relevant to separating weather-driven cooling loads and probabilistic residential forecasting. ScienceDirect |
| Lin & Wu (2021), “Residential Electric Load Forecasting via Attentive Transfer of Graph Neural Networks.” DOI: 10.24963/IJCAI.2021/374 | Transfer learning between apartments. IJCAI |
| Shi et al. (2025), “A short-term residential load forecasting framework based on dynamic spatio-temporal graph neural networks.” DOI: 10.1016/j.enbuild.2025.116311 | Advanced cross-household forecasting and dynamic household relationships. ScienceDirect |

What I would actually implement first

For Lodha Palava, I would not start with LSTM/GNN/Transformer simply because they are sophisticated. The technically defensible sequence is:

Data QA → 5-min appliance analytics → 15-min fused thermal/weather dataset → weather-normalized apartment benchmark → thermal-response models → LightGBM global prediction → probabilistic forecasting → LSTM/TFT comparison → transfer/GNN experimentation.

With your particular dataset, the three most differentiated outputs would be (1) weather-normalized Expected vs Actual apartment energy, (2) apartment-specific AC/thermal effectiveness, and (3) a global personalized model predicting next-day apartment and appliance loads. Those outputs use the unique combination of appliance submetering, indoor sensing, weather and apartment geometry instead of producing another conventional residential-energy dashboard.