The operation schedules and vampire-load analysis should not just be dashboard features; they should become measured-data-derived inputs to the residential simulation model.

The overall research design you are proposing is technically strong, but I would structure it so that you can separate occupant/appliance behaviour effects from physical apartment effects such as floor and orientation.

1. Recommended overall framework

I suggest calling the deliverable:

Residential Appliance Operation Schedule, Standby Load and Simulation Validation Framework

It would have four linked stages:

Stage 1 — Measured apartment calibration
Use one apartment for which you have good-quality EMD + indoor temperature/RH + outdoor weather data. Derive its actual appliance schedules and standby loads, insert them into the simulation, and calibrate the model against measured consumption and indoor environmental conditions.

Stage 2 — Ten-apartment schedule analytics
Derive appliance-specific schedules and vampire/standby loads for 10 monitored apartments independently.

Stage 3 — Representative averaged operating schedule
Convert those ten individual schedules into statistically representative appliance schedules.

Stage 4 — Physics-normalized apartment comparison
Apply the same representative schedule to apartment simulation variants while changing floor, orientation, exposure and other physical characteristics. Compare simulated consumption with the actual analytics results of the corresponding monitored apartments.

That gives you an important separation:

\[ \text{Measured Energy} = \text{Physical Building Effect} + \text{Weather Effect} + \text{Occupant/Appliance Behaviour} + \text{Equipment Efficiency} + \text{Residual} \]

Your simulation experiment can progressively isolate these effects.

2. Why deriving schedules from your monitored data is preferable

Residential energy simulation is particularly sensitive to occupant schedules and appliance behaviour.

NREL's Building America methodology explicitly uses hourly profiles for appliances, miscellaneous electrical loads, occupancy and lighting in residential simulation. It also distinguishes loads that are approximately continuous, such as refrigerators and transformers, from loads that are strongly time-dependent, such as cooking and dishwashing. The Department of Energy's Energy.gov

More recent NREL residential work goes further and develops 15-minute stochastic occupancy and appliance schedules, including Markov-chain approaches to represent residential behaviour instead of assuming a fixed schedule. NREL

This is particularly relevant to Lodha because you do not need to use generic U.S. residential schedules. You have measured Indian apartment data at five-minute resolution, so the generic schedules should only be used as methodological references.

3. Two schedules should actually be produced

I would not produce only one "operation schedule."

For every major appliance, produce:

A. Deterministic simulation schedule

A normalized 0–1 schedule representing average appliance load.

This is appropriate for:

- EnergyPlus internal gains;

- annual energy simulation;

- average cooling-load effects;

- comparing physical apartment variants.

B. Probabilistic/event schedule

Represent:

- probability of appliance starting;

- likely start-time range;

- duration distribution;

- number of events/day;

- probability of simultaneous appliance operation.

This is useful for:

- peak demand;

- stochastic simulation;

- load forecasting;

- demand response;

- realistic household behaviour.

This distinction matters because averaging ten washing-machine schedules, for example, could produce:

0.06 load from 08:00–20:00

even though no washing machine actually operates continuously like that.

For annual EnergyPlus internal-gain calculation, that fractional schedule may be acceptable. For appliance demand/peak simulations it is physically unrealistic.

4. Appliance operation schedule extraction methodology

For apartment \(i\), appliance \(a\), and five-minute interval \(t\):

\[ P_{i,a,t} \]

is your measured EMD power.

First identify appliance operating states:

\[ State_{a,t} = \begin{cases} 0 & \text{OFF}\\ 1 & \text{Standby}\\ 2 & \text{Idle/Low}\\ 3 & \text{Active} \end{cases} \]

Not every appliance requires all four states.

For example:

| Appliance | Recommended state model |
| --- | --- |
| Lighting | OFF / ON |
| Split AC | OFF / Standby / Compressor ON |
| Geyser | OFF / Heating |
| Refrigerator | Compressor OFF / Compressor ON |
| Washing machine | OFF / Standby / Operating cycle |
| Oven | OFF / Standby / Cooking |
| Plug circuits | Base/standby / Active |
| Fans | OFF / operating states if individually measurable |

This approach is well supported by appliance state-identification research. Adaptive clustering and Hidden Markov methods are commonly used to determine appliance working states from active-power sequences. ScienceDirect

Because your appliances are already separately submetered, your problem is actually easier than conventional NILM: you mainly need robust operating-state classification rather than disaggregation.

5. Do not use one arbitrary ON/OFF threshold

For example:

AC ON if kW > 0.1

would be too simplistic.

Instead derive an appliance-specific threshold from its measured distribution.

A good sequence is:

Clean data → remove impossible spikes → correct zero offset → cluster power states → identify OFF/standby/active distributions.

Potential algorithms:

- 1-D K-means;

- Gaussian Mixture Model;

- density-based clustering;

- histogram peak detection;

- Hidden Markov Model where cycling behaviour matters.

Recent appliance analytics research also uses adaptive household-specific thresholds obtained from historical data rather than one fixed threshold for all households. ScienceDirect

6. What I already see in Lakeside K 502

The Lakeside sample demonstrates why this approach is needed.

For example, some lighting circuits show highly persistent low-power states:

- Bed1 Lights has a strong cluster at approximately 0.0044–0.0048

- Living & Terrace Lights has a strong cluster around 0.0063–0.0069

assuming the recorded measurement unit is kW, those correspond to roughly 4–5 W and 6–7 W continuous residual loads.

However, I would not immediately label those values as vampire consumption.

They could represent:

- electronic drivers;

- another load on the same circuit;

- smart switches;

- plug loads;

- metering offset;

- genuine standby consumption.

This distinction should be incorporated into the platform as:

Candidate Standby / Continuous Residual Load

until circuit mapping or field verification confirms it.

7. Definition of vampire load

A useful technical definition is:

electricity consumed while equipment remains connected to the supply but is switched off or is not performing its primary service.

Berkeley Lab uses essentially this definition for standby/"vampire" electricity. Energy Technologies Area

Research has found the cumulative effect can be substantial. Measurements across ten California homes found whole-house standby power ranging from 14–169 W, averaging 67 W, although these historic figures should not be treated as expected values for Lodha. LBL ETA Publications

Another field study covering 75 Canadian homes estimated average standby energy at 427 kWh/year for the sampled homes. Again, this is a reference illustrating significance, not a benchmark for Indian flats. ScienceDirect

8. Very important: distinguish vampire load from baseload

The platform should separate these terms.

| Category | Example | Vampire? |
| --- | --- | --- |
| Refrigerator compressor cycling | Refrigerator | No |
| Router operating continuously | Router | No — always-on service |
| AC controller drawing power while AC is OFF | Split AC | Yes |
| TV electronics when TV is OFF | TV | Yes |
| Washing-machine electronics when unused | Washing machine | Yes |
| Geyser actively maintaining tank temperature | Water heater | Generally operational/standby thermal loss, separate category |
| Metering offset | Sensor | No |
| Unknown 5 W residual on lighting circuit | Unknown | Candidate only |

Otherwise your platform could considerably overstate vampire consumption.

9. Proposed vampire-load calculation

For each appliance identify the standby-state power:

\[ P_{sb,a} = Median(P_a | State=Standby) \]

Standby operating hours:

\[ H_{sb,a} = N_{sb,a} \times \frac{5}{60} \]

for five-minute EMD data.

Then:

\[ E_{vamp,a} = \sum_{t\in Standby} P_{a,t}\Delta t \]

or approximately:

\[ E_{vamp,a} = P_{sb,a}\times H_{sb,a} \]

Annualized vampire consumption:

\[ E_{vamp,a,annual} = E_{vamp,a,observed} \times \frac{365}{N_{observed\,days}} \]

And:

\[ Vampire\ Share_a = \frac{E_{vamp,a}} {E_{total,a}} \times100 \]

At apartment level:

\[ E_{vamp,flat} = \sum_a E_{vamp,a} \]

10. Platform output for vampire loads

I suggest a table like this:

| Appliance | Active power | Standby power | Standby h/day | Standby kWh/year | % appliance energy | Confidence |
| --- | --- | --- | --- | --- | --- | --- |
| Living AC | — | — | — | — | — | High |
| Bedroom AC | — | — | — | — | — | High |
| Washing machine | — | — | — | — | — | High |
| Oven | — | — | — | — | — | Medium |
| Lighting circuit | — | — | — | — | — | Candidate |
| Refrigerator | — | N/A | — | N/A | N/A | Cycling load |

And then:

Apartment Vampire Load Summary

- Continuous standby demand: W

- Daily standby energy: kWh/day

- Annual standby energy: kWh/year

- Standby share of total electricity: %

- Maximum technically avoidable standby energy: kWh/year

The last value should only include actual avoidable standby devices, not refrigerators or necessary always-on loads.

11. Reference method for annual standby energy

Berkeley Lab's residential modeling documentation expresses annual miscellaneous equipment energy in terms of active-use hours plus standby hours:

\[ StandbyEnergy = (8760-H_{active})P_{standby} \]

with appliance active energy calculated separately. Home Energy Saver

This is directly adaptable to your measured data, except your \(H_{active}\) and \(P_{standby}\) will be empirically derived rather than assumed.

12. Operation Schedule — core output

For each appliance create a 96-bin daily profile, because 24 hours × 4 = 96 fifteen-minute periods.

Even though EMD is measured every 5 minutes, use:

- 5-minute analysis to identify appliance events;

- 15-minute schedules for simulation.

For appliance \(a\):

\[ S_{a,d,h} = \frac{\bar P_{a,d,h}} {P_{design,a}} \]

where:

- \(S\) = simulation schedule fraction, 0–1;

- \(\bar P\) = observed mean power in the interval;

- \(P_{design}\) = appliance design/operating power.

13. How should \(P_{design}\) be obtained?

Prefer in this order:

- OEM/rated power;

- measured stable active-state power;

- robust active-state P95/P99;

- maximum measured power only as last resort.

Do not use absolute maximum because the sample already shows isolated spikes that would make the entire normalized schedule artificially small.

14. Schedule output should be separated by day type

At minimum:

Weekday

Monday–Friday

Weekend

Saturday–Sunday

For some appliances it is useful to have:

- Monday–Thursday;

- Friday;

- Saturday;

- Sunday.

Research using more than 90,000 residential smart-thermostat datasets found day-of-week to be an important factor in residential occupancy schedules. ScienceDirect

15. Seasonal schedules

Since many of your apartments have 13 months of data, I would additionally generate:

Summer / cooling season

Monsoon

Winter / mild season

This is particularly important for:

- AC;

- geyser;

- refrigerator;

- potentially lighting.

Do not average AC operation across the entire year.

Instead:

\[ S_{AC} = f( Time,\; DayType,\; Season,\; T_{out} ) \]

AC is fundamentally weather- and behaviour-dependent.

16. AC schedules require a different approach

This is especially important for the simulation.

Do not model AC simply as:

AC load schedule = historical kW profile.

If the simulation already has a physical split-AC model, use measured data to derive:

AC availability schedule

\[ P(AC\,available|hour) \]

and preferably a behavioural control model:

\[ P(AC_{ON}) = \frac{1} {1+\exp[-(\beta_0+\beta_1T_{in}+\beta_2RH+\beta_3Hour+\ldots)]} \]

Research specifically shows that residential AC use can be modeled probabilistically using measured temperature and occupant operation behaviour and then integrated into EnergyPlus. Importantly, generic schedule assumptions can badly misrepresent residential cooling demand. ScienceDirect

For your first deterministic simulation, however, the measured availability profile is acceptable.

17. Recommended schedule parameters for every appliance

The platform should calculate:

| Parameter | Meaning |
| --- | --- |
| Start-time distribution | Typical time appliance begins operation |
| End-time distribution | Typical end |
| ON probability | Probability active in each interval |
| Active power | kW while operating |
| Standby power | kW in standby |
| Events/day | Number of daily operations |
| Mean duration/event | minutes |
| P10/P50/P90 duration | variability |
| Weekday schedule | 96 points |
| Weekend schedule | 96 points |
| Seasonal schedules | summer/monsoon/winter |
| Daily energy | kWh/day |
| Coincidence | overlap with other appliances |

This becomes a useful appliance behaviour database, not just a chart.

18. Example schedule output

For a washing machine the output might eventually look like:

| Time | Weekday ON probability | Weekend ON probability | Normalized simulation fraction |
| --- | --- | --- | --- |
| 00:00–06:00 | 0.01 | 0.01 | 0.00 |
| 06:00–09:00 | 0.08 | 0.12 | 0.06 |
| 09:00–12:00 | 0.16 | 0.27 | 0.18 |
| 12:00–18:00 | 0.10 | 0.16 | 0.11 |
| 18:00–22:00 | 0.07 | 0.09 | 0.06 |
| 22:00–00:00 | 0.02 | 0.03 | 0.01 |

The numbers above are illustrative; the actual profile must come from the ten monitored apartments.

19. Event-based schedules for washing machines/geysers/ovens

For these appliances, also calculate:

\[ P(Start=a\;at\;t) \]

and:

\[ P(Duration=d|Start) \]

Research into residential energy flexibility has specifically used actual household appliance-use behaviour to identify likely earliest/latest start times of flexible loads. ScienceDirect

This has a direct future application for your platform:

Flexible Appliance Window: 09:00–14:00

rather than merely saying:

Washing machine energy = 0.7 kWh/day.

20. Creating the average schedule from 10 apartments

This part needs care.

Do not average raw kW from the ten apartments.

That mixes:

- appliance capacity;

- apartment size;

- ownership;

- behaviour.

First normalize every apartment:

\[ S_{i,a,t} = \frac{P_{active,i,a,t}} {P_{design,i,a}} \]

Then calculate:

\[ \bar S_{a,t} = \frac{1}{N} \sum_{i=1}^{N}S_{i,a,t} \]

For your study:

\[ N=10 \]

Thus:

\[ \boxed{ \bar S_{a,t} = \frac{ S_{1,a,t}+\cdots+S_{10,a,t} }{10} } \]

This produces your:

Lodha Palava Representative Appliance Operation Schedule

21. Median should also be calculated

Because ten apartments are a relatively small sample and individual households may behave very differently, provide:

- P25;

- Median;

- Mean;

- P75.

For simulation I would test:

Base case

Median schedule

Representative average

Mean schedule

Low-use behaviour

P25

High-use behaviour

P75

That gives you an extremely useful behavioural sensitivity band.

22. Average schedule must preserve energy

After creating a normalized schedule, verify:

\[ E_{schedule} = P_{design} \sum_tS_t\Delta t \]

against:

\[ E_{measured,average} = \frac{1}{10} \sum_iE_{measured,i} \]

Ideally:

\[ \left| \frac{ E_{schedule}-E_{measured,average} }{ E_{measured,average} } \right| <5\% \]

before putting it into simulation.

This prevents smoothing/normalization from inadvertently changing annual appliance energy.

23. Standby should be separated from the operating schedule

For each appliance:

\[ P_a(t) = P_{standby,a}S_{standby,a}(t) + P_{active,a}S_{active,a}(t) \]

For many appliances:

\[ S_{standby}(t) = 1-S_{active}(t) \]

provided the appliance remains plugged in.

This means your simulation should ideally contain two components.

Appliance active load

Variable schedule.

Appliance standby load

Mostly constant or inverse-to-active schedule.

This makes the simulation output much more transparent.

24. Simulation workflow I recommend

Your proposed research experiment can be converted into this sequence.

Experiment 1 — Calibrate one real apartment

For the selected apartment:

Actual inputs

- actual geometry;

- actual orientation;

- actual floor;

- actual adjacency/exposure;

- actual weather;

- actual appliance inventory;

- actual measured appliance schedules;

- actual AC behaviour;

- measured internal temperature/RH.

Simulation produces:

- total electricity;

- AC electricity;

- appliance electricity;

- cooling load;

- indoor temperature;

- hourly profile.

Compare against measured analytics.

25. Calibration metrics

At minimum use:

\[ NMBE= \frac{\sum(M_i-S_i)} {\sum M_i}\times100 \]

and:

\[ CV(RMSE)= \frac{ \sqrt{ \frac{1}{N} \sum(M_i-S_i)^2 } } {\bar M} \times100 \]

ASHRAE Guideline 14 criteria commonly cited for calibrated models are approximately:

| Resolution | NMBE | CV(RMSE) |
| --- | --- | --- |
| Monthly | ±5% | ≤15% |
| Hourly | ±10% | ≤30% |

With your detailed monitored dataset, I would assess both energy and indoor temperature, not just annual kWh. Research using sub-hourly measured data has demonstrated that calibrating both energy and dynamic indoor temperatures gives a much stronger test of building-model validity. ScienceDirect

26. Experiment 2 — Replace actual schedule with 10-flat average

Keep everything else identical.

Only replace:

\[ Schedule_{actual} \rightarrow Schedule_{10-flat-average} \]

Then:

\[ \Delta E_{behaviour} = E_{sim,actualschedule} - E_{sim,averageschedule} \]

This tells you:

How much of the calibrated apartment's result is caused by its individual household behaviour rather than representative Palava behaviour?

That is a very interesting analytical output.

27. Experiment 3 — Physical variants using common behaviour

Now freeze the same averaged schedule.

Create apartment variants by changing:

- floor;

- primary orientation;

- external exposure;

- roof exposure if top floor;

- ground/adjacency condition if lower floor;

- window orientation;

- solar exposure.

Thus:

\[ Schedule_i(t)=\bar S(t) \]

for every apartment.

This means behaviour is deliberately controlled.

Any simulated differences are then primarily caused by:

\[ Orientation + Floor + Envelope + Solar + Weather \]

This is exactly what you want if your research question is:

How much difference should the physical position of an apartment make if household behaviour were the same?

28. Then compare simulation with actual analytics

For apartment \(i\):

\[ Residual_i = E_{Actual,i} - E_{Simulated,i,\ AverageSchedule} \]

Calculate:

\[ PerformanceRatio_i = \frac{E_{Actual,i}} {E_{Simulated,i,\ AverageSchedule}} \]

Interpret approximately:

PR ≈ 1.0

Actual consumption consistent with expected consumption under representative behaviour.

PR > 1.0

Higher-than-expected actual consumption.

Possible causes:

- heavier appliance usage;

- more AC operation;

- lower thermostat;

- more occupants;

- poorer AC efficiency;

- unmodeled load.

PR < 1.0

Lower-than-expected consumption.

Potentially:

- conservative behaviour;

- lower occupancy;

- reduced AC use;

- appliance efficiency.

I would call this:

Behaviour–Performance Residual

rather than directly calling it efficiency.

29. This gives you an extremely strong analytics–simulation loop

Your platform can eventually display:

\[ \boxed{ Actual \rightarrow Analytics \rightarrow MeasuredSchedule \rightarrow CalibratedSimulation \rightarrow RepresentativeSchedule \rightarrow PhysicalVariants \rightarrow Actual-vs-Expected } \]

That is much stronger than keeping analytics and simulation as separate workstreams.

30. One important correction to your proposed approach

You said you will:

change floor, orientation, direction to simulate the 10 apartments.

That is valid only if those ten apartments are physically comparable.

If they contain:

- 1 BHK;

- 2 BHK;

- 3 BHK;

- 47 m²;

- 83 m²;

- different building types/layouts,

then changing only orientation and floor in the same model is not sufficient.

From the list you provided, areas range roughly from 47 to 83 m², with 1-BHK, 2-BHK and 3-BHK units.

Therefore I recommend one of two approaches.

Preferred approach

Select 10 apartments belonging to the same/sufficiently similar typology.

For example, your B3 / 2-BHK Ultima apartments around 68.4–68.7 m² appear to form a strong comparison group.

Alternative

Build separate prototype geometries:

- 1 BHK;

- 2 BHK Optima;

- 2 BHK Ultima;

- 3 BHK.

Then apply the representative schedule appropriate to each typology.

Otherwise the model residual will incorrectly attribute geometry/area differences to occupant behaviour.

31. Apartment averaging should ideally be stratified

Ultimately I suggest building schedules at two levels.

Level 1 — Palava general schedule

\[ S_{Palava,a,t} \]

all selected flats.

Level 2 — Flat-type schedule

For example:

\[ S_{2BHK,a,t} \]

and:

\[ S_{3BHK,a,t} \]

If enough observations exist, later also:

\[ S_{2BHK,weekday,summer,t} \]

This produces a much more reusable schedule library.

32. Recommended database/output architecture

The analytics platform should maintain something like:

Apartment_Schedule

| Apartment | Appliance | Season | DayType | Time | ScheduleFraction |
| --- | --- | --- | --- | --- | --- |

Appliance_States

| Apartment | Appliance | ActivePower | StandbyPower | ONThreshold | MeanDuration |
| --- | --- | --- | --- | --- | --- |

Vampire_Loads

| Apartment | Appliance | Standby_W | Standby_hr_day | kWh_year | Confidence |
| --- | --- | --- | --- | --- | --- |

Representative_Schedule

| Appliance | Season | DayType | Time | P25 | Median | Mean | P75 |
| --- | --- | --- | --- | --- | --- | --- | --- |

This dataset can directly feed the simulation workflow.

33. Recommended platform visuals

For Operation Schedule, I would show:

Appliance Operating Probability — Hour of Day

Heatmap:

appliance × hour

and another:

day of week × hour.

Also show:

- average start time;

- average duration;

- events/day;

- weekday/weekend difference;

- seasonal difference.

For Vampire Load, use:

Active / Standby / Off energy

stacked bar for each appliance.

Then:

Total Apartment Standby Demand

XX W

Annual Vampire Energy

XX kWh/year

Share of Apartment Electricity

X.X%

Avoidable Standby Energy

XX kWh/year

34. Research foundation for this deliverable

These are particularly relevant references.

Wilson et al. / NREL Building America House Simulation Protocols — provides normalized residential equipment, refrigerator, clothes washer, dryer, range/oven and MEL schedules for building simulation and explicitly separates operating and standby energy. The Department of Energy's Energy.gov

NREL End-Use Load Profiles / ResStock — develops residential end-use profiles at 15-minute resolution and uses stochastic occupancy models to associate occupancy activities with appliance schedules. ResStock

Diao et al. (2017), Energy and Buildings, “Modeling energy consumption in residential buildings: A bottom-up analysis based on occupant behavior pattern clustering and stochastic simulation.” The study demonstrates that clustered occupant schedules can differ materially from generic standard schedules and validates bottom-up residential models. ScienceDirect

Nijhuis et al. (2016), Energy and Buildings, “Bottom-up Markov Chain Monte Carlo approach for scenario based residential load modelling.” Develops residential load curves from appliance, behavioural and climate data and validates against measured smart-meter data. ScienceDirect

Ren et al., “Modelling and simulating occupant behaviour on air conditioning in residential buildings.” Uses measured data, logistic regression and stochastic AC behaviour within EnergyPlus and demonstrates why generic cooling schedules can cause major prediction errors. ScienceDirect

Wu et al. (2021), Energy and Buildings, adaptive clustering + FHMM. Provides a good methodological basis for identifying appliance operating states from measured power sequences. ScienceDirect

Ross & Meier / Lawrence Berkeley National Laboratory — standby power measurement. Provides the conceptual and measurement foundation for household standby/vampire-load analysis. LBL ETA Publications

Fung et al. (2003), Energy and Buildings, “Standby power requirements of household appliances in Canada.” Field-measured appliance standby electricity in 75 homes. ScienceDirect

ASHRAE Handbook Chapter 19 — supports deriving typical day-type schedules from long-term measured data and using NMBE/CV(RMSE) for model calibration. ASHRAE Handbook

Recommended final research architecture

I would therefore make the overall Lodha study three interconnected deliverables:

Deliverable 1 — Residential Data Analytics
Actual energy, appliances, weather, indoor environment, prediction, thermal/cooling performance.

Deliverable 2 — Appliance Schedule & Vampire Load Library
Measured operating states → appliance event analytics → 15-minute schedules → weekday/weekend/season schedules → P25/P50/mean/P75 → standby/vampire loads → EnergyPlus-ready files.

Deliverable 3 — Simulation & Validation Study
Individual measured schedule calibration → representative 10-apartment schedule → floor/orientation/exposure variants → simulation vs actual analytics → physical-effect and behaviour-residual analysis.

The key methodological improvement is that the first simulation should use the actual measured schedule, while the second-stage simulations deliberately use the same ten-apartment representative schedule. That makes the second experiment a controlled test of floor/orientation/physical effects, rather than an uncontrolled comparison of ten different household behaviours.